Industrial EKG

MCM Portable Kit Specifications

June 2010

Instrument Power

Line voltage						85-256 VAC

Current						200 ma, max

Frequency						47 - 64 Hz

Voltage Connections

Max input Voltage ~ Motor Line Voltage		500 V (line-to-line)

Min input Voltage ~ Motor Line Voltage		120 V

Connection type					3, Large alligator clip or user-supplied

Test Leads						MIL 3-pin to Female test lead banana plug

10 ft / 3 m length, alligator clips supplied

Current Connections

Max continuous MCM input current			5 A

Current transformers					3, Included

Ratio							Selectable 100, 200, 300, or 400 : 5

Motor lead wire diameter				1.56 in / 40 mm

Test leads						MIL 6-pin to open spade screw terminal

for CTs, 10 ft / 3 m length

Communication						RS422-to-Ethernet converter, included

10 ft / 3 m serial cable, included

10 ft / 3 m Ethernet crossover cable, incl

Alarm Indicator Light	 / Relay				Normal = OFF; N/C or N/O programmable

Software

Operating Systems					Windows XP Pro, Vista, Win/7 [Win7/64 in

XP/SP3 compat mode], MAC or Linux 									emulators may work but are not supported

MCMConfigure					Included on CD (unlicensed)

MCMScada-TCP/IP					Included on CD (license required)

MCMScada-Demo (no communication)		Included on CD (unlicensed), data viewer

Drivers for Ethernet communication 		None required.

Packaging

Type							Pelican 1510 wheeled travel case

Shipping Weight					37 lbs / 16.5 kg

Shipping Size						22 x 14 x 9.50 in / 56 x 34 x 24 cm